LIQUID PARAFFIN IP/BP/USP/EP
HIGH PURITY, MULTI-USE PARAFFIN OIL
Manufactured by: Sidhe Petrochemicals, Gujarat
GMP & GLP Certified | FDA-Approved Facility | Halal & Kosher Certified Facility

Sidhe Petrochemicals proudly presents Liquid Paraffin IP/BP/USP/EP, a high-purity, versatile Paraffin Oil suitable for Pharmaceutical, Personal Care, Skin Care and direct food contact application. Produced at our state-of-the-art GMP (Good Manufacturing Practices) and GLP (Good Laboratory Practices) certified facility, our LLP IP/BP/USP/EP complies with the highest global quality benchmarks.

APPLICATIONS IN PHARMACEUTICALS, COSMETICS, PERSONAL CARE, SKIN CARE & HAIR CARE PRODUCTS

LLP IP extensively used as bases for Pharmaceutical, Skin Care, Personal Care & Hair Care Products.

- Pharmaceuticals: Used in manufacturing of Laxative and for some ophthalmic formulations for dry eye relief.

- Food Industry: Used as Protective Coatings for Fruits & Vegetables to extend shelf life and also used as release agent to prevent sticking in baking or candy molds.

- Personal Care, Skin Care & Hair Care: Forms a barrier to lock in moisture in products like lotions, Creams, and baby oil.

PRODUCT SPECIFICATIONS (As per IP 2022)

| Sr No | Characteristic | Unit | Test Method | Specification |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Max |
| 1 | Description |  | Visual | A transparent, colorless, Oily liquid, Free from fluorescence by daylight,odourless |  |
| 2 | Color, Saybolt |  | ASTM D 156 | 30 |  |
| 3 | Kinematic Viscosity @ 40°C | cst | ASTM D 445/ ASTM D 7042 | 68 | 72 |
| 4 | Dynamic Viscosity @ 20° | mPa.s | IP/BP/EP | 110 | 230 |
| 5 | Weight per ml @ 25°C | gms/ml | IP/USP | 0.86 | 0.904 |
| 6 | Acidity or Alkalinity |  | IP/USP/BP/EP | The solution is colorless & NMT 0.1 ML of 0.1 M sodium Hydroxide is required to change of the color of the solution to pink |  |
| 7 | Light Absorption (Absorbance in the Range of 240-280 nm od 2% w/v solution in 2,2,4-Trimethylpentane) |  | IP |  | 0.1 |
| 8 | Readily Carbonisable Substances |  | IP/USP/BP/EP | Pass |  |
| 9 | Solid Paraffins |  | IP/USP/BP/EP | Pass |  |
| 10 | Sulphur Compounds |  | IP/USP | Pass |  |

REGULATORY COMPLIANCE & SAFETY STANDARDS

- Manufactured under FDA-approved GMP & GLP conditions.

- Halal and Kosher certified facility.

- Conforms to IP 2022 specifications.

STORAGE & PACKAGING

- Available in HDPE Drums and bulk loading option is also available.

- Shelf Life: More than two years from date of manufacture under cool, dry storage conditions, away from direct sunlight. Retesting is recommended after two years.

WHY CHOOSE SIDHE PETROCHEMICALS?

✔ GMP & GLP Certified Manufacturing Facility
✔ Consistent Purity & Compliance with IP Standards
✔ Stringent Quality Control & Analytical Testing
✔ Regulatory Certifications (Halal, Kosher, FDA)
✔ Prompt Customer Support & Reliable Supply Chain

For inquiries, technical specifications, or samples, please contact:
Mobile: +91- 9561190699 [Kunal Kumar Singh]
Email: kunalk@sidhepetrochemical.com